Document No. 9-28-01-0A

Title : Analog follow of iG5 with real analog output

Date : 9/28/01

Drive model : SV XXX iG5 - X

IG5 has the real analog output. The specification of this hardware is 1mA, 0 ~ 10V output. This is very useful for several purpose in the field. For example, the system need 2 motor but speed is different. 1st motor speed is changed from 0Hz to 60hz and 2nd motor speed range is 0Hz to 45Hz proportionally by first motor speed. The customer needs only one analog speed reference.

- Related parameters

| Code | Description | Keypad Display | Setting Range | Units | Factory Default | Setting Value |
| --- | --- | --- | --- | --- | --- | --- |
| DRV-04 | Frequency Mode (Freq. setting method) | Frq | 0 [Keypad-1] | - | (V1) 2 | (V1) 2 |
|  |  |  | 1 (Keypad-2) |  |  |  |
|  |  |  | 2 (V1) |  |  |  |
|  |  |  | 3 (I) |  |  |  |
|  |  |  | 4 (V1+I) |  |  |  |
|  |  |  | 5 (RS485) |  |  |  |
| I/O-01 | Filtering Time Constant for V1 Signal Input | I 1 | 0 to 9999 [ms] | 1 | 500 [ms] | 100[ms] |
| I/O-02 | V1 Input Minimum Voltage | I 2 | 0 to I/O-04 | 0.01 | 0.00 [V] | - |
| I/O-03 | Frequency corresponding to V1 Input Minimum Voltage | I 3 | 0 to FU1-20 | 0.01 | 0.00 [Hz] | - |
| I/O-04 | V1 Input Maximum Voltage | I 4 | I/O-02 to 12 [V] | 0.01 |  | 10[V] |
| I/O-05 | Frequency corresponding to V1 Input Maximum Voltage | I 5 | 0 to FU1-20 | 0.01 | 60.00 [Hz] | - |
| I/O-40 | FM (Frequency Meter) Output selection | I 40 | 0 (Frequency) | - | Frequency 0 | 2 (Voltage) |
|  |  |  | 1 (Current) |  |  |  |
|  |  |  | 2 (Voltage) |  |  |  |
|  |  |  | 3 (DC link Vtg) |  |  |  |
| I/O-41 | FM Output Adjustment | I 41 | 10 to 200 [%] | 1 | 100 [%] | - |

Master drive

VR V1 CM FM CM

Motor

Slave drive

CM   VI

Motor

2. Wire Diagram

Fig 1. Wiring Diagram

3. The performance data through test

A. Delay time between master drive and slave drive – it depends on the setting values of  I/O01.

2nd Drive current Waveform

1st Drive current waveform

Figure 2. Delay time between master and slave drives

495mS

B. Error between master drive speed and slvae drive speed

As in the figure 3, the maximum speed difference is less than 1%.

Figure 3.