DeviceNet

User Manual

- Option Board

- for SV-iS5 Series

Read this manual carefully before using the DeviceNet Option Board and follow the instructions exactly.

After reading this manual, keep it at handy for future reference.

- LG Industrial Systems

Thank you for purchase of LG DeviceNet Option Board!

SAFETY PRECAUTIONS

- Always follow safety precautions to prevent accidents and potential hazards from occurring.

- Safety precautions are classified into “WARNING” and “CAUTION” in this manual.

Indicates a potentially hazardous situation which, if not avoided, can result in serious injury or death.

WARNING

CAUTION

Indicates a potentially hazardous situation which, if not avoided, can result in minor to moderate injury, or serious damage to the product.

- Throughout this manual we use the following two illustrations to make you aware of safety considerations:

Identifies potential hazards under certain conditions.

Read the message and follow the instructions carefully.

Identifies shock hazards under certain conditions.

Particular attention should be directed because dangerous voltage may be present.

- Keep this manual at handy for quick reference.

CAUTION

- Be sure to take ESD (Electrostatic Discharge) protection measures when you touch the board.

Otherwise, the option board may get damaged due to static charges.

- Implement wiring change on the Option Board after checking that the power supply is off.

Otherwise, there is a danger of connecting error and damage to the board.

- Be sure to fix inverter to option board tightly.

Otherwise, there is a danger of connecting error and damage to the board.

- Be sure to install a termination resistor (120ohm, 1/4W) at the end of the network.

- Be sure to check parameter unit when setting parameters.

Otherwise, there is a danger of connecting error.

# Introduction

By using a DeviceNet communication board, SV-iS5 inverter can be connected to a DeviceNet network.

## Through LG DeviceNet Option Board…

- Inverter can be controlled and monitored by PLC sequence program or any Master Module.

- Multiple inverters can be connected to one communication cable with simple and easy installation, dramatically saving wiring, maintenance cost and time.

- Compatible with PC System, PLC and any controllers is available, making Factory Automation more easily.

### Vocabulary

In this manual we refer to the:

- DeviceNet Option Board for iS5 series as the Option Board

- iS5 series inverter as the inverter

## Kit Contents

The DeviceNet option board kit consists of:

- DeviceNet Option Board, 1 pcs

- 5 pin Connector, 1 pcs

- Mounting poles, 3 pcs

- Installation Manual

## DeviceNet Option Board Specification

- Device Type: AC Drives

- Explicit Peer to Peer Messaging: Support

- I/O Peer to Peer Messaging: N/A (Not Available)

- Configuration Consistency Value: N/A

- Faulted Node Recovery (Off-Line): Support

- Baud Rate Support: 125, 250, 500 (kbps)

- Master/Scanner (Predefined M/S Connection): Support

- I/O Slave Messaging
  - Polling: Support
  - Bit Strobe, Cyclic, COS (Change of State): N/A

- Range of Input Voltages: 11 – 25V DC

## Installing the Communication Card

### Board Layout

DeviceNet

Connector

Status LED

Inverter

Connector

MS

NS

Figure 1 – Board layout

### Status LED

| MS (Module Status) LED | Checks the status of incoming power to option board, CPU function and communication to the Inverter. |
| --- | --- |
| NS (Network Status) LED | Checks the connection of option card on the Network and DeviceNet power status. |

Note) Refer to Chapter 4 - TROUBLESHOOTING for more details.

### DeviceNet Connection Terminal

| DeviceNet Terminal Block | Terminal | Signal | Function | Cable Color |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 | 1 | ##### Common | Common | Black |
|  | 2 | CAN Low | Signal Low | Blue |
|  | 3 | Shield | ##### Shield | Bare |
|  | 4 | CAN High | Signal High | White |
|  | 5 | V+ | Power supply (11-24VDC) | Red |

Figure 2 – DeviceNet Terminal Block

# Installing the Option Board

## Installing the Option Board to the Inverter

Note) DeviceNet Option Board can be installed to the inverter having software V 1.05 and later.

Figure 3 – Installing the DeviceNet Board to the Inverter

### Setting the DeviceNet Parameter

#### Setting the MAC ID <COM Group, #10>

- MAC ID (Media Access Control Identifier, Station Number) is a unique value to identify nodes in the DeviceNet Network. When setting, you must ensure that the each serial device on the network has a unique address.

- Setting and changing MAC ID is available via keypad.

- The factory default setting is DeviceNet address 63. If DPRAM communication between Inverter and the Option card becomes faulty, the value will default to Station Number 63.

|  | Min Value | Max Value | Parameter Setting |
| --- | --- | --- | --- |
| MAC ID | 0 | 63 | COM Group, #10 |

- When MAC ID is changed during Run, Option board is automatically reset to find the device new MAC ID is allocated on the network. If a duplicate Mac ID is checked, NS (Network Status) LED turns Solid Red. In this case, it should be changed to different MAC ID. In the normal operation, LED color is flashing Green.

#### Setting the Baud Rate < COM Group, #11>

| Baud Rate | Trunk Cable Length |  |  | Drop Length |  |
| --- | --- | --- | --- | --- | --- |
|  | Thick Cable | Thin Cable |  | Max Length | Total |
| 125 kbps | 500 m (1640 ft.) |  | 100 m (328 ft.) | 6 m (20 ft.) | 156 m (512 ft.) |
| 250 kbps | 250 m (820 ft.) |  |  |  | 78 m (256 ft.) |
| 500 kbps | 100 m (328 ft.) |  |  |  | 39m (128ft.) |

Table 1 – Baud Rate vs. Cable Length

- The Baud Rate determines the maximum length of the DeviceNet cable. Refer to Table 1 to determine cable lengths and baud rates. The above is only met when DeviceNet-dedicated cables are used. For more information on the DeviceNet cables, makers and detail specifications, refer to the Open DeviceNet Vendor Association (ODVA) homepage at “http://www.odva.org/”.

- NS LED remains OFF when the setting of actual Network communication speed and Baud Rate does not match.

- Set the desirable Baud Rate via Keypad. Changing this parameter does not change the actual data rate until power is cycled. Reset the inverter or request Reset Service to the Inverter Reset Identity Object.

- NS LED will turn to flashing Green when Network Baud Rate matches Baud Rate of the option card and a unique MAC ID is allocated.

#### Setting Assembly Instance

Assembly Instance has four types of Sending/Receiving data through Poll I/O communication. Refer to Assembly Object in Chapter 7 for detailed information.

## Setting Other Parameters

### Setting Option Mode [COM Group, #02]

| Via DeviceNet | COM Group, #02 setting |
| --- | --- |
| Issuing Operating command only | Command |
| Issuing Frequency command only | Freq |
| Issuing Operating + Frequency command | Cmd+Freq |

Table 2 – Setting Option Mode

Note) This setting supercedes setting in FU1 group.

### Setting TimeOut [I/O, #49]

The factory default setting for TimeOut is 1 sec. When communication between inverter and DeviceNet network is disrupted in the Run mode, inverter checks the communication failure and performs operation under the setting in I/O group #48 until Master-setting Time + preset TimeOut elapses.

## Poll I/O Connection

It is a Data transaction between Inverter and Scanner.

- Input/output size: 4 bytes

- Communication Rate: 0 (default)

- Data Transaction: Poll I/O

Data transaction thru Poll I/O is determined by the setting of Assembly Instance [COM, #12,13].

Assembly Instance consists of Input and Output, based on Scanner side. Therefore, Input Data means data Scanner receives. For inverter side, it is the feedback value to Scanner.

By contrast, Output Data is the data Scanner transmits to the Inverter as a new command.

Input Assembly

Scanner

(Ex:PLC)

Inverter

Output Assembly

Figure 4 - Data Transaction via Poll I/O Communication

|  | Scanner Side | Inverter Side |
| --- | --- | --- |
| Input Assembly Data | Receive data | Transmit data |
| Output Assembly Data | Transmit data | Receive data |

# Modes of Operation

## PowerUp Reset mode

During a powerup or reset, the option board:

- Performs powerup initialization. First Module Status LED flashes Green for 0.25sec to Red for 0.25sec and then turns to Solid Green when DPRAM is in normal operation.

- Then, Network Status LED flashes Green for 0.25sec to Red for 0.25sec

- After no duplicate MAC ID is detected, Network Status LED flashes Green, indicating the option board is successfully connected to the Network. However, communication with other nodes has not been initiated.

If the above steps is failed, follow the below steps. No action is required in the normal operating state.

- When DPRAM is not working properly, Module Status LED turns Solid Red. In this case, Check the option board’s connection with Inverter first and cycle the power.

- When NS remains OFF, not flashing Green,

  - Check the DeviceNet Power

  - Check the Baud Rate of Network and the Option board matches.

- When duplicate MAC ID error occurs, Network Status LED turns Solid Red. In this case, allocate the different MAC ID via Keypad.

- When the option board is in communication with other nodes, NS (Network Status) LED turns Solid Green.

.

## When EMC (Explicit Message Connection) is established by Scanner

- Network Status LED turns green. During this state, if EMC setting is deactivated, the LED will turn green after 10 seconds delay. Once EMC is connected, I/O Connection setting is available. At this time, the Network Status LED will not change.

- If I/O Connection is not established within given time, then Time Out will occur and Network Status LED turns flashing Red. (Depending on the time setting of EMC, the status can be changed to green again.)

# Troubleshooting

The current status of Device and Network can be monitored through built-in LED

(MS, NS).

[NS (Network Status) LED]

| LED | Status | Cause | Diagnostics |
| --- | --- | --- | --- |
| OFF | Off-Line (No Power) | The option card is not receiving power from the Network. | Check DeviceNet power and cable connections and the power connection on the DeviceNet terminal block. |
|  |  | Single node on the network | Check DeviceNet Master node operation for correct Communication. |
|  |  | Incorrect Baud Rate is set. | Change the baud rate setting and reset the inverter. |
| Flashing Green | On-Line, Not Connected | Communication is set after duplicate node check is finished, but connection to other node is not completed. | Normal operating status before user makes connection. |
| Solid Green | On-Line, Connected (Link OK) | More than one EMC connection is established. | Polled I/O connection is available. |
| Flashing Red | Connection Time-Out Critical Link Failure. | Polled I/O connection is Timed Out | Inverter Reset. Request Reset Service to the Inverter Reset Identity Object. Retry I/O connection. |
| Solid Red | Faults occurred | Duplicate MAC ID check failed | Change the setting of MAC ID. |
|  |  | Bus Off State | Check for line connection |
| Green Flashing Red | Self- diagnostic | Device is under self-diagnostic mode. | Wait for a moment. |
| Red  Flashing Green | Communication Fault | State of Communication Fault due to the failure to pass the Network Access. Identity communication Fault Request is accepted | No action required. |

[MS (Module Status) LED]

| LED | Status | Cause | Diagnostics |
| --- | --- | --- | --- |
| Off | No Power | 5V Power is not applied to the option card. | Check the incoming power to inverter is provided. Check 5V power is ON. |
| Solid Green | Operational | Normal operation state |  |
| Solid Red | Unrecoverable Fault | Data transaction through DPRAM is disrupted.. | Check the connection between option card and Inverter. |
| Flashing Green & Red | Self Test | Device is in self-test mode. |  |

- EDS(Electronic Data Sheet)

EDS files are specially formatted ASCII files that provide all of the information necessary for a configuration tool such as the DeviceNet Manager, to access and alter the parameters of a device. The EDS file contains information on the number of parameters in a device and how those parameters are grouped together. Information about each parameter is contained in this file such as parameter min, max, and default values, parameter data format and scaling and the parameter name and units.

Install EDS files for iS5 to control iS5 parameters using DeviceNet Manager program. This file can be downloaded from : http://www.lgis.com or contact your LG representative.

# DeviceNet Data Tables

- Message

- R: Read Only

- R/W: Read / Write enable

- <Device Profile>

- AC/DC Drives: 0x 02

- <Object Model>

| Object Class Name | Class Code |
| --- | --- |
| Identity Object | 0x01 |
| Message Router | 0x02 |
| DeviceNet | 0x03 |
| Assembly | 0x04 |
| Connection | 0x05 |
| Motor Data | 0x28 |
| Control Supervisor | 0x29 |
| AC/DC Drive | 0x2A |
| Inverter | 0x64 |

< Class 1 - Identity Object>

| Class Code | 0x01 |
| --- | --- |
| Instance | 1 (All attributes are instance 1) |

| Attribute ID | Attribute Name | Access Method |
| --- | --- | --- |
| 1 | Vendor ID | R |
| 2 | Device Type | R |
| 3 | Product Code | R |
| 4 | Revision | R |
|  | Major Revision (High Byte) |  |
|  | Minor Revision (Low Byte) |  |
| 5 | Status | R |
| 6 | Serial Number | R |
| 7 | Product Name | R |

| Service Name | Service Code | Implemented for: |  |
| --- | --- | --- | --- |
|  |  | Class | Instance |
| Get_Attribute_Single | 0x0E | No | Yes |
| Reset | 0x05 | No | Yes |
| Set_Attribute_Single | 0x10 | No | Yes |

< Class 3 - DeviceNet Object >

| Class Code | 0x03 |
| --- | --- |
| Instance | 1 (All attributes are instance 1) |

| Attribute ID | Attribute Name | Access Method |
| --- | --- | --- |
| 1 | MAC ID | R/W |
| 2 | Baud Rate | R/W |
| 3 | BOI | Not support |
| 4 | Bus-Off Counter | Not support |
| 5 | Allocation Information: Allocation Choice Byte Master’s MAC ID | R |
| 6 | MAC ID Switch Changed | R |
| 7 | Baud Rate Changed | Not support |
| 8 | MAC ID Switch Value | Not support |
| 9 | Baud Rate Switch Value | Not support |

| Service Name | Service Code | Implemented for: |  |
| --- | --- | --- | --- |
|  |  | Class | Instance |
| Get_Attribute_Single | 0x0E | Yes | Yes |
| Set_Attribute_Single | 0x10 | No | Yes |
| Allocate Master/Slave Connection Set | 0x4B | No | Yes |
| Release Group2 Identifier Set | 0x4C | No | Yes |

< Class 4 - Assembly Object >

| Class Code | 0x04 |
| --- | --- |
| Instance | 1 (All attributes are instance 1) |

| Service Name | Service Code | Implemented for: |  |
| --- | --- | --- | --- |
|  |  | Class | Instance |
| Get_Attribute_Single | 0x0E | No | Yes |
| Set_Attribute_Single | 0x10 | No | Yes |

< Output Assembly Data Attribute Format >

| Instance | Byte | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 (100) | 0 |  |  |  |  |  | Fault Reset |  | Run Fwd |
|  | 1 |  |  |  |  |  |  |  |  |
|  | 2 | Speed Reference (Low Byte) – RPM unit (Speed Reference (Low Byte) – Hz unit) |  |  |  |  |  |  |  |
|  | 3 | Speed Reference (High Byte) – RPM unit (Speed Reference (High Byte) – Hz unit) |  |  |  |  |  |  |  |
| 21 (101) | 0 |  | NetRef | NetCtrl |  |  | Fault Reset | Run Rev | Run Fwd |
|  | 1 |  |  |  |  |  |  |  |  |
|  | 2 | Speed Reference (Low Byte) – RPM unit (Speed Reference (Low Byte) – Hz unit) |  |  |  |  |  |  |  |
|  | 3 | Speed Reference (High Byte) – RPM unit (Speed Reference (High Byte) – Hz unit) |  |  |  |  |  |  |  |

| Name | Description | Related Attribute |  |
| --- | --- | --- | --- |
|  |  | Class | Attr. ID |
| Run Fwd | Forward Run Command | 0x29 | 3 |
| Run Rev | Reverse Run Command | 0x29 | 4 |
| Fault reset | Fault Reset Command | 0x29 | 12 |
| NetRef | Not used | 0x2A | 4 |
| NetCtrl 5 | Not used | 0x29 | 5 |
| Speed Reference | Speed Command | 0x2A | 8 |

< Input Assembly Data Attribute Format >

| Instance | Byte | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70 (110) | 0 |  |  |  |  |  | Run-ning1 |  | Faulted |
|  | 1 |  |  |  |  |  |  |  |  |
|  | 2 | Speed Reference (Low Byte) – RPM unit (Speed Reference (Low Byte) – Hz unit) |  |  |  |  |  |  |  |
|  | 3 | Speed Reference (High Byte) – RPM unit (Speed Reference (High Byte) – Hz unit) |  |  |  |  |  |  |  |
| 71 (111) | 0 | At Ref. | Ref from Net | Ctrl from Net | Ready | Run-ning 2, (Rev) | Run-ning 1, (Fwd) | Warn- ing | Faulted |
|  | 1 |  |  |  |  |  |  |  |  |
|  | 2 | Speed Reference (Low Byte) – RPM unit (Speed Reference (Low Byte) – Hz unit) |  |  |  |  |  |  |  |
|  | 3 | Speed Reference (High Byte) – RPM unit (Speed Reference (High Byte) – Hz unit) |  |  |  |  |  |  |  |

| Name | Description | Related Attribute |  |
| --- | --- | --- | --- |
|  |  | Class | Attr. ID |
| Faulted | DPRAM or Inverter Error | 0x29 | 10 |
| Warning | Not Supported | 0x29 | 11 |
| Running1 | Motor is running Forward | 0x29 | 7 |
| Running2 | Motor is running Reverse | 0x29 | 8 |
| Ready | Motor is ready to run | 0x29 | 9 |
| Ctrl From Net | Run/Stop control | 0x29 | 15 |
| Ref From Net | Speed control | 0x2A | 29 |
| At Reference | Reach at Reference Speed | 0x2A | 3 |
| Drive State | Current Motor State | 0x29 | 6 |
| Speed Actual | Speed Command | 0x2A | 7 |

< Class 5 - Connection Object >

| Class Code | 0x05 |  |
| --- | --- | --- |
| Instance | 1 | Predefined EMC |
|  | 2 | Poll I/O |
|  | 6, 7, 8, 9, 10 | Dynamic EMC |

| Attribute ID | Attribute Name | Access Method |  |
| --- | --- | --- | --- |
|  |  | I/O | EMC |
|  |  | Established/ Timed Out | Established/ Deferred delete |
| 1 | State | R | R |
| 2 | Instance_type | R | R |
| 3 | TransportClass_trigger | R | R |
| 4 | Produced_connection_id | R/W | R |
| 5 | Consumed_connection_id | R/W | R |
| 6 | initial_comm_characteristics | R | R |
| 7 | Produced_connection_size | R | R |
| 8 | Consumed_connection_size | R | R |
| 9 | Expected_packet_rate | R/W | R/W |
| 10 – 11 | N/A |  |  |
| 12 | Watchdog_timeout_action | R/W | R/W |
| 13 | Produced_connection_path_ length | R | R |
| 14 | Produced_connection_path | R | R |
| 15 | Consumed_connection_path_ length | R | R |
| 16 | Consumed_connection_path | R | R |
| 17 | Production_inhibit_time | R/W | R |

| Service Name | Service Code | Implemented for: |  |  |
| --- | --- | --- | --- | --- |
|  |  | Class | Instance |  |
| Get_Attribute_Single | 0x0E | No |  | Yes |
| Reset | 0x05 | No |  | Yes |
| Set_Attribute_Single | 0x10 | No |  | Yes |

<Class 28 - Motor Data Object >

| Class Code | 0x28 |
| --- | --- |
| Instance | 1 (All attributes are instance 1) |

| Attribute ID | Attribute Name | Access Method |
| --- | --- | --- |
| 3 | MotorType | R |
| 6 | RatedCurrent | R/W |
| 7 | RatedVoltage | R |

| Service Name | Service Code | Implemented for: |  |
| --- | --- | --- | --- |
|  |  | Class | Instance |
| Get_Attribute_Single | 0x0E | No | Yes |
| Set_Attribute_Single | 0x10 | No | Yes |

< Class 29 - Control Supervisor Object >

| Class Code | 0x29 |
| --- | --- |
| Instance | 1 (All attributes are instance 1) |

| Attribute ID | Attribute Name | Access Method |
| --- | --- | --- |
| 3 | Run 1 (Forward command) | R/W |
| 4 | Run 2 (Reverse command) | R/W |
| 5 | NetCtrl | R |
| 6 | State | R |
| 7 | Running1 (Forward running) | R |
| 8 | Running2 (Reverse running) | R |
| 9 | Ready | R |
| 10 | Faulted | R |
| 12 | FaultRst | R/W |
| 13 | FaultCode | R |
| 15 | CtrlFromNet | R |

| Service Name | Service Code | Implemented for: |  |
| --- | --- | --- | --- |
|  |  | Class | Instance |
| Get_Attribute_Single | 0x0E | No | Yes |
| Set_Attribute_Single | 0x10 | No | Yes |

< Class 2A - AC/DC Drive Object >

| Class Code | 0x2A |
| --- | --- |
| Instance | 1 (All attributes are instance 1) |

| Attribute ID | Attribute Name | Access Method |
| --- | --- | --- |
| 3 | AtReference | R |
| 4 | NetRef | R/W |
| 6 | DriveMode | R/W |
| 7 | SpeedActual | R |
| 8 | SpeedRef | R/W |
| 9 | CurrentActual | R |
| 29 | RefFromNet | R |
| 100 | Actual Hz | R |
| 101 | Reference Hz | R/W |
| 102 | Acc. Time | R/W |
| 103 | Dec. Time | R/W |

| Service Name | Service Code | Implemented for: |  |
| --- | --- | --- | --- |
|  |  | Class | Instance |
| Get_Attribute_Single | 0x0E | No | Yes |
| Set_Attribute_Single | 0x10 | No | Yes |

< Class 64 - Inverter Object >

| Class Code | 0x64 |  | Attribute Number |
| --- | --- | --- | --- |
| Instance | 1 | DRV Group | iS5 Parameter code # + 1 |
|  | 2 | FU1/FU2 Group | Same as iS5 Parameter code # |
|  | 3 | I/O Group | Same as iS5 Parameter code # |
|  | 4 | COM Group | Same as iS5 Parameter code # |
|  | 5 | APP Group | Same as iS5 Parameter code # |

Note) Refer to iS5 inverter manual for reference of Attribute Number. It is the same as iS5 Parameter Code Number.

| Service Name | Service Code |  | Implemented for: |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Class | Instance |  |
| Get_Attribute_Single |  | 0x0E | Yes |  | Yes |
| Set_Attribute_Single |  | 0x10 | No |  | Yes |

# Parameter Code (Hex)

<Common>

| Parameter Address | Parameter Name | Unit | Read/Write | Data Value (Hex) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0000 | Inverter Model | - | R | 4: SV-iS5 |  |  |  |  |  |
| 0x0001 | Applicable Motor Rating | - | R | 0: 0.75 4: 5.5 8: 18.5 C: 45 | 1: 1.5 5: 7.5 9: 22 D: 55 |  | 2: 2.2 6: 11 A: 30 E: 75 |  | 3: 3.7 7: 15 B: 37 F: 90 |
|  |  |  |  | 10:110 13: 200 16: 375 |  | 11: 132 14: 220 (Unit: kW) |  | 12: 160 15: 280 |  |
| 0x0002 | Inverter Input Voltage | - | R | 0: 220V 1: 440V |  |  |  |  |  |
| 0x0003 | Software version | - | R | 0100: Ver. 1.00, 0101: Ver 1.01 |  |  |  |  |  |
| 0x0005 | Reference Frequency | 0.01Hz | R/W |  |  |  |  |  |  |
| 0x0006 | Operation reference | - | R/W | Bit 0: Stop Bit 1: Forward Run Bit 2: Reverse Run Bit 3: Fault Reset Bit 4: Emergency Stop |  |  |  |  |  |
| 0x0007 | Accel time | 0.1 sec | R/W |  |  |  |  |  |  |
| 0x0008 | Decel time | 0.1 sec | R/W |  |  |  |  |  |  |
| 0x0009 | Output Current | 0.1 A | R |  |  |  |  |  |  |
| 0x000A | Output frequency | 0.01 Hz | R |  |  |  |  |  |  |
| 0x000B | Output voltage | 0.1 V | R |  |  |  |  |  |  |
| 0x000C | DC Link voltage | 0.1 V | R |  |  |  |  |  |  |
| 0x000D | Output power | 0.1 kW | R |  |  |  |  |  |  |
| 0x000E | Operation reference | - | R | Bit 0: Stop Bit 1: Forward Run Bit 2: Reverse Run Bit 3: Fault (Trip) Bit 4: Accelerating Bit 5: Decelerating Bit 6: Speed reached Bit 7: DC Braking Bit 8: Stopping Bit 9: Not Used Bit10: Brake Open Bit 11: FWD Run Command Bit 12: REV Run Command Bit13: Rem. Run/Stop Bit14: Rem. Freq. Cmd |  |  |  |  |  |
| 0x000F | Trip info | - | R | Bit 0: OCT1 Bit 1: OV Bit 2: EXT-A Bit 3: BX Bit 4: OCT2 Bit 5: GF Bit 6: OH Bit 7: ETH Bit 8: OLT Bit 9: HW-diag Bit 10: EXT-B Bit 11: FO Bit 12: OPT Bit 13: PO Bit 14: IOLT Bit 15: LV |  |  |  |  |  |
| 0x0010 | Input terminal info | - | R | Bit 0: P1 Bit 1: P2 Bit 2: P3 Bit 3: P4 Bit 4: P5 Bit 5: P6, Bit 6: RST Bit 7: BX Bit 8: JOG, Bit 9: FX Bit 10: RX |  |  |  |  |  |
| 0x0011 | Output terminal info | - | R | Bit 0: Q1 (OC1), Bit 1: Q2 (OC2) Bit 2: Q3 (OC3), Bit 3: AUX Bit 4: 30AC |  |  |  |  |  |
| 0x0012 | V1 | - | R | 0 – FFC0 |  |  |  |  |  |
| 0x0013 | V2 | - | R | 0 – FFC0 |  |  |  |  |  |
| 0x0014 | I | - | R | 0 – FFC0 |  |  |  |  |  |
| 0x0015 | RPM | - | R |  |  |  |  |  |  |

< DRV Group >

| Address | Parameter No. | Parameter Name | Default | Max value | Min value | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 5100 | DRV#00 | Cmd. freq | 0 | MaxFreq | 0 | 0.01Hz |
| 5101 | DRV#01 | Acc. Time | 100 | 6000 | 0 | 0.1sec |
| 5102 | DRV#02 | Dec. Time | 200 | 6000 | 0 | 0.1sec |
| 5103 | DRV#03 | Drive mode | 1 | 2 | 0 |  |
| 5104 | DRV#04 | Freq. mode | 0 | 4 | 0 |  |
| 5105 | DRV#05 | Step freq - 1 | 1000 | MaxFreq | StartFreq | 0.01Hz |
| 5106 | DRV#06 | Step freq - 2 | 2000 | MaxFreq | StartFreq | 0.01Hz |
| 5107 | DRV#07 | Step freq - 3 | 3000 | MaxFreq | StartFreq | 0.01Hz |
| 5108 | DRV#08 | Current | - | - | - | 0.1A |
| 5109 | DRV#09 | Speed | - | - | - | 1rpm |
| 510A | DRV#10 | DC Link Voltage |  | - | - | V |

< FU1 Group >

| Address | Parameter No. |  | Parameter Name | Default | Max value | Min value | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5203 |  | FU1 #03 | Run prohibit | 0 | 2 | 0 |  |
| 5205 |  | FU1 #05 | Acc. pattern | 0 | 4 | 0 |  |
| 5206 |  | FU1 #06 | Dec. pattern | 0 | 4 | 0 |  |
| 5207 |  | FU1 #07 | Stop mode | 0 | 2 | 0 |  |
| 5208 |  | FU1 #08 | DcBr freq. | 500 | 6000 | StartFreq | 0.01Hz |
| 5209 |  | FU1 #09 | DcBlk time | 10 | 6000 | 0 | 0.01 sec |
| 520A |  | FU1 #10 | DcBr value | 50 | 200 | 0 | % |
| 520B |  | FU1 #11 | DcBr time | 10 | 600 | 0 | 0.1sec |
| 520C |  | FU1 #12 | DcSt value | 50 | 200 | 0 | % |
| 520D |  | FU1 #13 | DcSt time | 0 | 600 | 0 | 0.1sec |
| 5214 |  | FU1 #20 | Max freq. | 6000 | 40000 | 4000 | 0.01Hz |
| 5215 |  | FU1 #21 | Base freq. | 6000 | MaxFreq | 3000 | 0.01Hz |
| 5216 |  | FU1 #22 | Start freq. | 50 | 6000 | 1 | 0.01Hz |
| 5217 |  | FU1 #23 | Freq limit | 0 | 1 | 0 |  |
| 5218 |  | FU1 #24 | F-limit Lo. | 50 | HighFreq | StartFreq | 0.01Hz |
| 5219 |  | FU1 #25 | F-limit Hi. | 6000 | MaxFreq | LowFreq | 0.01Hz |
| 521A |  | FU1 #26 | Torque boost | 0 | 1 | 0 |  |
| 521B |  | FU1 #27 | Fwd boost | 20 | 150 | 0 | 0.1% |
| 521C |  | FU1 #28 | Rev boost | 20 | 150 | 0 | 0.1% |
| 521D |  | FU1 #29 | V/F pattern | 0 | 2 | 0 |  |
| 521E |  | FU1 #30 | User freq. 1 | 1500 | MaxFreq | 0 | 0.01Hz |
| 521F |  | FU1 #31 | User volt. 1 | 25 | 100 | 0 | % |
| 5220 |  | FU1 #32 | User freq. 2 | 3000 | MaxFreq | 0 | 0.01Hz |
| 5221 |  | FU1 #33 | User volt. 2 | 50 | 100 | 0 | % |
| 5222 |  | FU1 #34 | User freq. 3 | 4500 | MaxFreq | 0 | 0.01Hz |
| 5223 |  | FU1 #35 | User volt. 3 | 75 | 100 | 0 | % |
| 5224 |  | FU1 #36 | User freq. 4 | 6000 | MaxFreq | 0 | 0.01Hz |
| 5225 |  | FU1 #37 | User volt. 4 | 100 | 100 | 0 | % |
| 5226 |  | FU1 #38 | Volt control | 1000 | 1100 | 400 | 0.1% |
| 5227 |  | FU1 #39 | Energy save | 0 | 30 | 0 | % |
| 5232 |  | FU1 #50 | ETH select | 0 | 1 | 0 |  |
| 5233 |  | FU1 #51 | ETH 1min | 180 | 200 | ETH Cont | % |
| 5234 |  | FU1 #52 | ETH Cont | 100 | 150 | 50 | % |
| 5235 |  | FU1 #53 | Motor type | 0 | 1 | 0 |  |
| 5236 |  | FU1 #54 | OL level | 150 | 150 | 30 | % |
| 5237 |  | FU1 #55 | OL time | 100 | 300 | 0 | 0.1sec |
| 5238 |  | FU1 #56 | OLT select | 1 | 1 | 0 |  |
| 5239 |  | FU1 #57 | OLT level | 180 | 200 | 30 | % |
| 523A |  | FU1 #58 | OLT time | 600 | 600 | 0 | 0.1sec |
| 523B |  | FU1 #59 | Stall prev. | 0 | 7 | 0 |  |
| 523C |  | FU1 #60 | Stall level | 180 | 250 | 30 | % |

< FU2 Group >

| Address | Parameter No. | Parameter Name | Default | Max value |  | Min value | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5307 | FU2 #07 | Dwell freq | 500 |  | MaxFreq | StartFreq |  | 0.01Hz |
| 5308 | FU1 #08 | Dwell time | 0 |  | 100 | 0 |  | 0.1sec |
| 530A | FU2 #10 | Jump freq | 0 |  | 1 | 0 |  |  |
| 530B | FU2 #11 | Jump lo 1 | 1000 |  | Jump Hi 1 | StartFreq |  | 0.01Hz |
| 530C | FU2#12 | Jump Hi 1 | 1500 |  | MaxFreq | Jump Lo 1 |  | 0.01Hz |
| 530D | FU2 #13 | Jump lo 2 | 2000 |  | Jump Hi 2 | StartFreq |  | 0.01Hz |
| 530E | FU2 #14 | Jump Hi 2 | 2500 |  | MaxFreq | Jump Lo 2 |  | 0.01Hz |
| 530F | FU2 #15 | Jump lo 3 | 3000 |  | Jump Hi 3 | StartFreq |  | 0.01Hz |
| 5310 | FU2 #16 | Jump Hi 3 | 3500 |  | MaxFreq | Jump Lo 3 |  | 0.01Hz |
| 5311 | FU2 #17 | Start Curve | 40 |  | 100 | 1 |  | % |
| 5312 | FU2 #18 | End Curve | 40 |  | 100 | 1 |  | % |
| 5313 | FU2 #19 | Trip select | 0 |  | 3 | 0 |  | BIT |
| 5314 | FU2 #20 | Power-on run | 0 |  | 1 | 0 |  |  |
| 5315 | FU2 #21 | RST restart | 0 |  | 1 | 0 |  |  |
| 5316 | FU2 #22 | Speed Search | 0 |  | 15 | 0 |  | BIT |
| 5317 | FU2 #23 | SS Sup-Curr | 100 |  | 200 | 80 |  |  |
| 5318 | FU2 #24 | SS P-gain | 100 |  | 9999 | 0 |  |  |
| 5319 | FU2 #25 | SS I-gain | 1000 |  | 9999 | 0 |  |  |
| 531A | FU2 #26 | Retry number | 0 |  | 10 | 0 |  |  |
| 531B | FU2 #27 | Retry delay | 10 |  | 600 | 0 |  | 0.1sec |
| 531E | FU2#30 | Motor select | 0 |  | 9 | 0 |  |  |
| 531F | FU2#31 | Pole number | 4 |  | 12 | 2 |  |  |
| 5320 | FU2 #32 | Rated-Slip | Depends on motor |  | 1000 | 0 |  | 0.01Hz |
| 5321 | FU2 #33 | Rated-Curr |  |  | 2000 | 10 |  | 0.1A |
| 5322 | FU2 #34 | Noload-Curr |  |  | 2000 | 5 |  | 0.1A |
| 5324 | FU2 #36 | Efficiency |  |  | 100 | 70 |  | % |
| 5325 | FU2 #37 | Inertia rate | 0 |  | 1 | 0 |  |  |
| 5327 | FU2 #39 | Carrier freq | 50 |  | 150 | 10 |  | 0.1kHZ |
| 5328 | FU2 #40 | Control mode | 0 |  | 2 | 0 |  |  |
| 5329 | FU2 #41 | Auto tuning | 0 |  | 1 | 0 |  |  |
| 532A | FU2 #42 | Rs | Depends on motor |  | 5000 | 0 |  | 0.001 ohm |
| 532B | FU2 #43 | Rr |  |  | 5000 | 0 |  | 0.001 ohm |
| 532C | FU2 #44 | Lsigma |  |  | MaxInduc | 0 |  | 0.001 mH |
| 532D | FU2 #45 | SL P-gain | 32767 |  | 32767 | 0 |  |  |
| 532E | FU2 #46 | SL I-gain | 3276 |  | 32767 | 0 |  |  |
| 532F | FU2 #47 | proc PI mode | 0 |  | 1 | 0 |  |  |
| 5330 | FU2 #48 | PID Ref | 1 |  | 1 | 0 |  |  |
| 5331 | FU2 #49 | PID Ref Mode | 0 |  | 5 | 0 |  |  |
| 5332 | FU2 #50 | PID Out Dir | 1 |  | 1 | 0 |  |  |
| 5333 | FU2 #51 | PID F/B | 0 |  | 2 | 0 |  |  |
| 5334 | FU2 #52 | PID P-gain | 3000 |  | 9999 | 0 |  | 0.1% |
| 5335 | FU2 #53 | PID I-time | 10 |  | 320 | 0 |  | 0.1sec |
| 5336 | FU2 #54 | PID D-time | 0 |  | 9999 | 0 |  | 0.1 msec |
| 5337 | FU2 #55 | PID +limit | 6000 |  | MaxFreq | 0 |  | 0.01Hz |
| 5338 | FU2 #56 | PID -limit | 6000 |  | MaxFreq | 0 |  | 0.01Hz |
| 5339 | FU2 #57 | PID Out Inv | 0 |  | 1 | 0 |  |  |
| 533A | FU2 #58 | PID OutScale | 1000 |  | 9999 | 1 |  | 0.1% |
| 533B | FU2 #59 | PID P2-gian | 1000 |  | 9999 | 0 |  | 0.1% |
| 533C | FU2 #60 | P-gain Scale | 1000 |  | 1000 | 0 |  | 0.1% |
| 5345 | FU2 #69 | Acc/Dec ch F | 0 |  | MaxFreq | 0 |  | 0.01Hz |
| 5346 | FU2 #70 | Acc/Dec freq | 0 |  | 1 | 0 |  |  |
| 5347 | FU2 #71 | Time scale | 1 |  | 2 | 0 |  |  |
| 5348 | FU2 #72 | PowerOn disp | 0 |  | 12 | 0 |  |  |
| 5349 | FU2 #73 | User disp | 0 |  | 2 | 0 |  |  |
| 534A | FU2 #74 | RPM factor | 100 |  | 1000 | 1 |  | % |
| 534B | FU2 #75 | DB mode | 1 |  | 2 | 0 |  |  |
| 534C | FU2 #76 | DB %ED | 10 |  | 30 | 0 |  | % |
| 5351 | FU2 #81 | 2nd Acc time | 50 |  | 6000 | 0 |  | 0.1sec |
| 5352 | FU2 #82 | 2nd Dec time | 100 |  | 6000 | 0 |  | 0.1sec |
| 5353 | FU2 #83 | 2nd BaseFreq | 6000 |  | maxFreq | 3000 |  | 0.01Hz |
| 5354 | FU2 #84 | 2nd V/F | 0 |  | 2 | 0 |  |  |
| 5355 | FU2 #85 | 2nd F-boost | 20 |  | 150 | 0 |  | 0.1% |
| 5356 | FU2 #86 | 2nd R-boost | 20 |  | 150 | 0 |  | 0.1% |
| 5357 | FU2 #87 | 2nd Stall | 150 |  | 150 | 30 |  | % |
| 5358 | FU2 #88 | 2nd ETH 1min | 180 |  | 200 | 2nd ETH Cont |  | % |
| 5359 | FU2 #89 | 2nd ETH Cont. | 120 |  | 2nd ETH 1min | 50 |  | % |
| 535A | FU2 #90 | 2nd R-Curr | 36 |  | 2000 | 10 |  | 0.1A |
| 535D | FU2 #93 | Para. Init | 0 |  | 8 | 0 |  |  |

< I/O Group >

| Address | Parameter No. | Parameter Name | Default | Max value |  | Min value | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5401 | I/O #01 | V1 filter | 10 |  | 9999 | 0 |  | ms |
| 5402 | I/O #02 | V1 volt x1 | 0 |  | V1 vort x2 | 0 |  | 0.01V |
| 5403 | I/O #03 | V1 freq y1 | 0 |  | MaxFreq | 0 |  | 0.01Hz |
| 5404 | I/O #04 | V1 volt x2 | 1000 |  | 1000 | V1 volt x1 |  | 0.01V |
| 5405 | I/O #05 | V1 freq y2 | 6000 |  | MaxFreq | 0 |  | 0.01Hz |
| 5406 | I/O #06 | I filter | 10 |  | 9999 | 0 |  | ms |
| 5407 | I/O #07 | I curr x1 | 400 |  | I curr x2 | 0 |  | 0.01mA |
| 5408 | I/O #08 | I freq y1 | 0 |  | MaxFreq | 0 |  | 0.01Hz |
| 5409 | I/O #09 | I curr x2 | 2000 |  | 2000 | I curr x1 |  | 0.01mA |
| 540A | I/O #10 | I freq y2 | 6000 |  | MaxFreq | 0 |  | 0.01Hz |
| 540B | I/O #11 | Wire broken | 0 |  | 2 | 0 |  |  |
| 540C | I/O #12 | P1 define | 0 |  | 32 | 0 |  |  |
| 540D | I/O #13 | P2 define | 1 |  | 32 | 0 |  |  |
| 540E | I/O #14 | P3 define | 2 |  | 32 | 0 |  |  |
| 5411 | I/O #17 | Ti Filt Num | 15 |  | 50 | 2 |  |  |
| 5414 | I/O #20 | Jog freq | 1000 |  | MaxFreq | StartFreq |  | 0.01Hz |
| 5415 | I/O #21 | Step freq - 4 | 4000 |  | MaxFreq | StartFreq |  | 0.01Hz |
| 5416 | I/O #22 | Step freq - 5 | 5000 |  | MaxFreq | StartFreq |  | 0.01Hz |
| 5417 | I/O #23 | Step freq - 6 | 4000 |  | MaxFreq | StartFreq |  | 0.01Hz |
| 5418 | I/O #24 | Step freq - 7 | 3000 |  | MaxFreq | StartFreq |  | 0.01Hz |
| 5419 | I/O #25 | Acc time– 1 | 200 |  | 6000 | 0 |  | 0.1sec |
| 541A | I/O #26 | Dec time – 1 | 200 |  | 6000 | 0 |  | 0.1sec |
| 541B | I/O #27 | Acc time – 2 | 300 |  | 6000 | 0 |  | 0.1sec |
| 541C | I/O #28 | Dec time – 2 | 300 |  | 6000 | 0 |  | 0.1sec |
| 541D | I/O #29 | Acc time – 3 | 400 |  | 6000 | 0 |  | 0.1sec |
| 541E | I/O #30 | Dec time - 3 | 400 |  | 6000 | 0 |  | 0.1sec |
| 541F | I/O #31 | Acc time – 4 | 500 |  | 6000 | 0 |  | 0.1sec |
| 5420 | I/O #32 | Dec time – 4 | 500 |  | 6000 | 0 |  | 0.1sec |
| 5421 | I/O #33 | Acc time – 5 | 400 |  | 6000 | 0 |  | 0.1sec |
| 5422 | I/O #34 | Dec time – 5 | 400 |  | 6000 | 0 |  | 0.1sec |
| 5423 | I/O #35 | Acc time – 6 | 300 |  | 6000 | 0 |  | 0.1sec |
| 5424 | I/O #36 | Dec time – 6 | 300 |  | 6000 | 0 |  | 0.1sec |
| 5425 | I/O #37 | Acc time – 7 | 200 |  | 6000 | 0 |  | 0.1sec |
| 5426 | I/O #38 | Dec time – 7 | 200 |  | 6000 | 0 |  | 0.1sec |
| 5428 | I/O #40 | FM mode | 0 |  | 3 | 0 |  |  |
| 5429 | I/O #41 | FM adjust | 100 |  | 200 | 10 |  | % |
| 542A | I/O #42 | FDT freq | 3000 |  | MaxFreq | 0 |  | 0.01Hz |
| 542B | I/O #43 | FDT band | 1000 |  | MaxFreq | 0 |  | 0.01Hz |
| 542C | I/O #44 | Aux mode | 12 |  | 23 | 0 |  |  |
| 542D | I/O #45 | Relay mode | 2 |  | 7 | 0 |  | BIT3 |
| 542E | I/O #46 | Inv No. | 1 |  | 31 | 1 |  |  |
| 542F | I/O #47 | Baud rate | 3 |  | 4 | 0 |  |  |
| 5430 | I/O #48 | Lost command | 0 |  | 2 | 0 |  |  |
| 5431 | I/O #49 | Time out | 10 |  | 1200 | 1 |  | 0.1sec |

Note) If you need to know specific parameter addresses for Auto Sequence Operation, contact LG representatives.

< EXT Group >

| Address | Parameter No. | Parameter Name | Default | Max value |  | Min value | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5501 | EXT #01 | Sub B/D |  |  |  |  |  |  |
| 5502 | EXT #02 | P4 define | 3 |  | 32 | 0 |  |  |
| 5503 | EXT #03 | P5 define | 4 |  | 32 | 0 |  |  |
| 5504 | EXT #04 | P6 define | 5 |  | 32 | 0 |  |  |
| 5505 | EXT #05 | V2 mode | 0 |  | 2 | 0 |  |  |
| 5506 | EXT #06 | V2 filter | 10 |  | 9999 | 0 |  | msec |
| 5507 | EXT #07 | V2 volt x1 | 0 |  | V2 volt x2 | 0 |  | 0.01V |
| 5508 | EXT #08 | V2 freq y1 | 0 |  | MaxFreq | 0 |  | 0.01Hz |
| 5509 | EXT #09 | V2 volt x2 | 1000 |  | 1000 | V2 volt x1 |  | 0.01V |
| 550A | EXT #10 | V2 freq y2 | 6000 |  | MaxFreq | 0 |  | 0.01Hz |
| 550E | EXT #14 | F mode | 0 |  | 2 | 0 |  |  |
| 550F | EXT #15 | F pulse set | 0 |  | 1 | 0 |  |  |
| 5510 | EXT #16 | F pulse num | 1024 |  | 4096 | 360 |  |  |
| 5511 | EXT #17 | F filter | 10 |  | 9999 | 0 |  | msec |
| 5512 | EXT #18 | F pulse x1 | 0 |  | 1000 | 0 |  | 0.1kHz |
| 5513 | EXT #19 | F freq y1 | 0 |  | MaxFreq | 0 |  | 0.01Hz |
| 5514 | EXT #20 | F pulse x2 | 100 |  | 1000 | 0 |  | 0.1kHz |
| 5515 | EXT #21 | F freq y2 | 6000 |  | MaxFreq | 0 |  | 0.01Hz |
| 5516 | EXT #22 | PG P-gain | 3000 |  | 9999 | 0 |  |  |
| 5517 | EXT #23 | PG I-gain | 300 |  | 9999 | 0 |  |  |
| 5518 | EXT #24 | PG Slip Freq | 100 |  | 200 | 0 |  | % |
| 551E | EXT #30 | Q1 define | 0 |  | 23 | 0 |  |  |
| 551F | EXT #31 | Q2 define | 1 |  | 23 | 0 |  |  |
| 5520 | EXT #32 | Q3 define | 2 |  | 23 | 0 |  |  |
| 5522 | EXT #34 | LM mode | 1 |  | 3 | 0 |  |  |
| 5523 | EXT #35 | LM adjust | 100 |  | 200 | 10 |  | % |
| 5528 | EXT #40 | AM1 mode | 0 |  | 3 | 0 |  |  |
| 5529 | EXT #41 | AM1 adjust | 100 |  | 200 | 10 |  | % |
| 552A | EXT #42 | AM2 mode | 3 |  | 3 | 0 |  |  |
| 552B | EXT #43 | AM2 adjust | 100 |  | 200 | 10 |  | % |

< COM Group >

| Address | Parameter No. | Parameter Name | Default | Max value |  | Min value | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5601 | COM #01 | Opt B/D |  |  | 7 | 0 |  |  |
| 5602 | COM #02 | Opt mode |  |  | 3 | 0 |  |  |
| 5603 | COM #03 | Opt version | 1.00 |  |  |  |  |  |

Note) Setting Inverter Station # and Baud rate: I/O-46, 47

- COM-01 [Opt B/D]

Indicates Option boards installed. This value is automatically set when the boards are installed..

- COM-02 [Opt Mode]

Determines whether Run/Stop/Reference Frequency is set via Communication.

| Value | Display | Description |
| --- | --- | --- |
| 0 | None | Disabled |
| 1 | Command | Run/Stop setting via Communication |
| 2 | Freq | Frequency setting via Communication |
| 3 | Cmd + Freq | Run/Stop/Reference Frequency via Communication |

- COM-03 [Opt Version]

Displays version of Option Board.

< APP Group >

| Address | Parameter No. |  | Parameter Name | Default | Max value |  |  | Min value | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5701 |  | APP #01 | APP mode | 0 |  | 3 | 0 |  |  |  |
| 5702 |  | APP #02 | Trv. Amp[%] | 0 |  | 200 | 0 |  |  | 0.1% |
| 5703 |  | APP #03 | Trv. Scr | 0 |  | 500 | 0 |  |  | 0.1% |
| 5704 |  | APP #04 | Trv Acc Time | 20 |  | 6000 | 1 |  |  | 0.1sec |
| 5705 |  | APP #05 | Trv Dec Time | 30 |  | 6000 | 1 |  |  | 0.1sec |
| 5706 |  | APP #06 | Trv Off Hi | 0 |  | 200 | 0 |  |  | 0.1% |
| 5707 |  | APP #07 | Trv Off Lo | 0 |  | 200 | 0 |  |  | 0.1% |
| 5708 |  | APP #08 | Aux Mot Run | - |  | - | - |  |  |  |
| 5709 |  | APP #09 | Starting Aux | 1 |  | 4 | 1 |  |  |  |
| 570A |  | APP #10 | Auto Op Time | - |  | - | - |  |  |  |
| 570B |  | APP #11 | Start freq1 | 4999 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 570C |  | APP #12 | Start freq2 | 4999 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 570D |  | APP #13 | Start freq3 | 4999 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 570E |  | APP #14 | Start freq4 | 4999 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 570F |  | APP #15 | Stop freq1 | 1500 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 5710 |  | APP #16 | Stop freq2 | 1500 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 5711 |  | APP #17 | Stop freq3 | 1500 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 5712 |  | APP #18 | Stop freq4 | 1500 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 5713 |  | APP #19 | Aux start DT | 600 |  | 9999 | 0 |  |  | 0.1sec |
| 5714 |  | APP #20 | Aux stop DT | 600 |  | 9999 | 0 |  |  | 0.1sec |
| 5715 |  | APP #21 | Nbr Aux’ | 4 |  | 4 | 0 |  |  |  |
| 5716 |  | APP #22 | Regul Bypass | 0 |  | 1 | 0 |  |  |  |
| 5717 |  | APP #23 | Sleep Delay | 600 |  | 9999 | 0 |  |  | 0.1sec |
| 5718 |  | APP #24 | Sleep Freq | 19 |  | MaxFreq | 0 |  |  | 0.01Hz |
| 5719 |  | APP #25 | WakeUp level | 35 |  | 100 | 0 |  |  | 1% |
| 571A |  | APP #26 | AutoCh_Mode | 1 |  | 2 | 0 |  |  |  |
| 571B |  | APP #27 | AutoEx intv | 4320 |  | 5940 | 0 |  |  | 0.1sec |
| 571C |  | APP #28 | AutoEx level | 20 |  | 100 | 0 |  |  | 1% |
| 571D |  | APP #29 | Inter-lock | 0 |  | 1 | 0 |  |  |  |
| 571E |  | APP #30 | Actual Value | - |  | - | - |  |  | 0.01Hz |
| 571F |  | APP #31 | Actual Perc | - |  | - | - |  |  | 1% |
| 5720 |  | APP #32 | Draw mode | 0 |  | 3 | 0 |  |  |  |
| 5721 |  | APP #33 | DrawPerc | 100 |  | 150 | 0 |  |  | 1% |

LG

LG Industrial Systems Co., Ltd.

LGIS constantly endeavors to improve its product so that

information in this manual is subject to change without notice.

Visit Our Website: http://www.lgis.com/

May, 2002

Publication #: 10310000365

Status Attribute

Bit number0 (Owned)8 (Recoverable Minor Fault)Other BitsMeaningConnected to the masterDPRAM ErrorNot support

Range of Mac ID: 0 to 63

Baud Rate

Value012Baud Rate125 kbps250 kbps500 kbps

Allocation Choice Byte

76543210Not SupportedPolledExplicit Message

Setting Reference Control and Run/Strop Control can Only be done via LCD Keypad. Therefore, NetRef, NetCtrl in Instance 21 and 101 is not available.

MotorType Attribute Squirrel Cage Induction Motor: #7

NetCtrl Attribute:  This Attribute setting determines the control location for the motor. This value only can be set through the keypad for the safety reason. Changing this via DeviceNet does not cause error and cannot affect the setting.

NetRef Attribute

This setting is only done via Keypad for safety reason. Changing this via DeviceNet does not cause error and cannot affect the setting.

The changed value in Common affects the current setting but returns to the previous setting when power is cycled or Inverter is reset. However, changing value is immediately reflected in other parameter groups even in the case of Reset or Power On/Off.

Depends on connected motor.

Depends on connected motor.

Depends on connected motor.

Run/Stop Setting Address - Use 0x0006 in Common

Freq Setting Address - Use 0x0005 in Common