* How to Make RS232 connection cable directly to iS5 unit

iS5 keypad connector

Computer

(serial port)

1. Layout of inverter unit keypad terminal pin

2 4 6 8 10

. . . . .

. . . . .

(Top view)

1 3 5 7 9

| keypad Pin number | keypad pin name | computer RS232 (Dip 9 pin) | RS232 name | Description |
| --- | --- | --- | --- | --- |
| 1 | 5V |  |  | 5V power supply |
| 2 | -15V |  |  | LCD bright regulation power |
| 3 | 5G(GND) | Pin 5 | GND | Power ground |
| 4 | TXEN |  |  | RS485 communication “Send Enable” signal |
| 5 | 5G(GND) |  |  | Power ground |
| 6 | TXD | Pin 2 | RXD | RS232 sending signal pin |
| 7 | CHKL |  |  | Keypad connection check |
| 8 | RXD | Pin 3 | TXD | RS232 receiving signal pin |
| 9 | ALM |  |  | Fault signal pin |
| 10 | 5V |  |  | 5V power supply |

- RS232 connection

Remove keypad and put the RS232 connection cable to keypad terminal. Use three pins for this direct RS232 connection. Following is the connection. Only three pins are useful.

| Inverter keypad terminal | Computer DIP 9 pinconnector | Description |
| --- | --- | --- |
| 6(TXD) | 2(RXD) | Send/Receive signal |
| 8(RXD) | 3(TXD) | Send/Receive signal |
| 3(GND) | 5(GND) | Ground |