# Introduction

This manual describes the specifications, handling instructions, and programming information for G4F-HO1A/HD1A, G6F–HO1A/HD1A. It is the high-speed counter module used with CPU of the GLOFA PLC GM/4/6 series and the MASTER-K200S/300S series.

The G3F-HSCA, the G4F-HSCA and the G6F-HSCA consist of 24bit counting part which performs increment or decrement count, an external input part, a comparison part which compares the set value specified by the user with the current counting value, and an output part which outputs the comparison results.

High Speed Counter Module has functions as below:

1) 1 Phase

(1) Up/Down counter by program.

(2) Up/Down counter by phase B.

2) 1,2,4 multiple of phase up/down counter for 2 phase.

3) Up/Down counter by phase A/phase B for CW/CCW.

4) Preset / Gate by program and the external terminal.

5) 7 coincidence output conditions and 4 transistor outputs per channel.

6) 5 functions (Count clear, Count latch, Sampling counter, Pulse frequency counter, Periodic pulse counter)