6.8 Common Resource

The following explains about common resource of XGT-Panel.

6.8.1 Message management

Manages the message contents of a message tag.

• Adds, deletes, or edits the message contents which is related on a message tag.

• Only the message contents which are registered in message management menu can be used by a message tag.

• The registered messages can be used by a multiple message tags.

• The available range of ID : 0 - 65535

• Max. size of content : Up to 24 letters

• The total number of message content varies according to the memory usage.

6.8.2 Image Management

Manages bitmap images.

< Adds bitmap image files >

(1) Click the Browse button. then, choose a image file to register.

(2) Click the Preview button.

(3) Specify a ID. Then click the Add button to register a image.

6.8.3 Parameter

Specifies various parameters of Panel Editor.

1) Initial screen

• Specifies a screen to display when the XGT-Panel is turned on.

• If the initial screen is not specified, XGT-Panel displays a screen which has lowest ID

2) Backlight off

• Turns the Backlight off if there is not a key input during specified time.(0 ~ 65535minutes)

• If any Key is pushed when the backlight is in the off-state, the backlight is turned on again.

3) Screen saver

Changes a current screen to the specified screen if there is not a key input during specified time.

4) Latch

• Specifies the latch area of system memory of XGT Panel.

• The data of latch area are retained when power is turned off(B type only)

5) Online screen change

• Changes a current screen according to the value of device.

• Refer to Ch. 6.7.2 digit tag for details about specifying address.

6) Send current screen No.

• Sends a number of current screen to PLC.

• Refer to Ch. 6.7.2 digit tag for details about specifying address.

6.8.4 Alarm

• Specifies various properties about alarm function.

• Displays a registered alarm messages and operates a buzzer when a specified activating condition occurs.

• Click the Add button to register a alarm

1) Address

• Specifies the address of device(Bit device)

• Refer to Ch. 6.7.5 button tag for details about specifying address.

2) Alarm trigger

• Off : Operates at the rising edge of a value of device

• On : Operates at the falling edge of a value of device

3) Buzzer action

| Item | Description |
| --- | --- |
| None | Buzzer does not operates. |
| Buzzer On | Buzzer operates during specified time. (The buzzer stops operation if the ESC/ALM key is pushed while it operates) |
| Permanently | Buzzer operates until the ESC/ALM key is pushed |

4) Alarm message

Max. size of content : Up to 37 letters.

<The example of alarm>

6.8.5 Calculation

Specifies a calculation function.

• Click the Add button to register a calculation function

1) Trigger condition

Specifies a activating condition of calculation function.

| Condition | Description |
| --- | --- |
| Always | Operates at every scan |
| On | Operates once at the rising edge of a value of trigger device |
| Off | Operates once at the falling edge of a value of trigger device |
| Toggle | Operates at the rising or falling edge of a value of trigger device |
| During On | Operates at every scan during value of trigger device is On |
| During Off | Operates at every scan during value of trigger device is Off |

2) Trigger Address

• Specifies the address of trigger device(Bit device)

• Refer to Ch. 6.7.5 button tag for details about specifying address.

3) Operand Address

• Specifies the address of operand devices(Word device, Constant is available)

• Refer to Ch. 6.7.2 digit tag for details about specifying address.

4) Operator

• Specifies a operator.

| Operator | Description |  |
| --- | --- | --- |
| + | Adds operand 2 to operand 1 |  |
| - | Subtracts operand 2 from operand 1 |  |
| x | multiplies operand 2 with operand 1. |  |
| / | divides operand 1 by operand 2 |  |
| % | Gets the remainder when operand 1 is divided by operand 2 |  |
| & | Compares each bit of operand 1 to the corresponding bit of the operand 2. If both are 1, the corresponding bit of result device is set to 1. Otherwise it is set to 0. |  |
| ¦ | Compares each bit of operand 1 to the corresponding bit of the operand 2. If either bit is 1, the corresponding result bit is set to 1. Otherwise it is set to 0. |  |
| ^ | Compares each bit of operand 1 to the corresponding bit of the operand 2. If one bit is 0 and the other bit is 1, the corresponding bit of result device is set to 1. Otherwise it is set to 0 |  |
| <<, >> | Shift the operand 1 left(<<) or right(>>) by the number of positions the operand 2 specifies |  |
| < | Compare the operand1 to the operand 2 to test the validity of the specified relationship. | Writes 1 to the result device if the tested relationship is true and writes 0 if it is false. |
| > |  |  |
| == |  |  |

5) Result Address

• Specifies the address of device to save the result of calculation.(Word device)

• Refer to Ch. 6.7.2 digit tag for details about specifying address.

6) Data

• Specifies the data type.

7) Display format

(1) DEC : Displays by decimal format.

(2) HEX : Displays by hexadecimal format

(3) BCD : Displays by binary coded decimal format

< Example of calculation >

6.8.6 Block communication

Specifies properties of block communication function.

• Click the Add button to register a block communication function

1) Trigger condition

Specifies a activating condition of block communication.

| Condition | Description |
| --- | --- |
| Always | Operates at every scan |
| On | Operates once at the rising edge of a value of trigger device |
| Off | Operates once at the falling edge of a value of trigger device |
| Toggle | Operates at the rising or falling edge of a value of trigger device |
| During On | Operates at every scan during value of trigger device is On |
| During Off | Operates at every scan during value of trigger device is Off |

2) Trigger Address

• Specifies the address of trigger device(Bit device)

• Refer to Ch. 6.7.5 button tag for details about specifying address.

3) Direction

• HMI <- PLC : Reads values of specified devices of PLC and writes them into the system memory of HMI.

• PLC <- HMI : Reads values of the system memory of HMI and writes them into specified devices of PLC.

4) Data

Specifies a size of data to communicate.(Max. 60Words)

5) PLC Address & HMI Address

• Specifies the addresses of PLC and system memory of HMI.

• Refer to Ch. 6.7.2 digit tag for details about specifying address.

6.8.7 Reservation

Turns a specified bit on at a specified time(B type only)

• Click the Add button to register a reservation function

1) Trigger condition

Specifies a activating condition of reservation function.

| Condition | Description |
| --- | --- |
| Always | Operates always |
| During On | Operates during value of trigger device is On |
| During Off | Operates during value of trigger device is Off |

2) Trigger Address

• Specifies the address of trigger device(Bit device)

• Refer to Ch. 6.7.5 button tag for details about specifying address.

3) Repeat period

| Item | Repeat Period |
| --- | --- |
| Once | Operates once at the time to be reserved |
| Every day(D) | Operates at the specified time of every day |
| Every week(W) | Operates at the specified time of day of the week. |
| Every month(M) | Operates at the specified time of the day of the month. |

4) Run Address and Action

• Specifies the address of device and the kind of action(On or Off)

• Refer to Ch. 6.7.5 button tag for details about specifying address

• Assume that reservation function is specified as shown above

• M0000(Bit device) of PLC is turned on at 9:00:30 of first day of every month.