Appendix 1. System Memory

| Address | Name | Description | Remark |
| --- | --- | --- | --- |
| 0~899 | User memory |  |  |
| 900 | RTC_C_Y | Year of RTC | BCD |
| 901 | RTC_MON | Month of RTC | BCD |
| 902 | RTC_DAY | Day of RTC | BCD |
| 903 | RTC_HOUR | Hour of RTC | BCD |
| 904 | RTC_MIN | Minute of RTC | BCD |
| 905 | RTC_SEC | Second of RTC | BCD |
| 906 | RTC_WEEK | The day of week (Sunday:0 – Saturday:6 ) | BCD |
| 907 | SCAN_MIN | Stores the minimum scan time | ms |
| 908 | SCAN_MAX | Stores the maximum scan time | ms |
| 909 | SCAN_CUR | Stores the current scan time | ms |
| 910 | HMI_TYPE | Stores the type of XGT Panel(A Type:0, B Type:1) |  |
| 911 | HMI_VER | Stores the version of XGT Panel |  |
| 912 | VER_CY | Stores the year of O/S |  |
| 913 | VER_MD | Stores the month and date of O/S |  |
| 914 | CUR_SCR | Stores the number of current screen |  |
| 915 | RESERVED | - |  |
| 916 | COM1_COUNT | Stores the error count of channel 1 |  |
| 917 | COM1_ERROR | Stores the current error code of channel 1 |  |
| 918 | COM2_COUNT | Stores the error count of channel 2 |  |
| 919 | COM2_ERROR | Stores the current error code of channel 2 |  |
| 920 | KEY_ESC | Stores the status of ESC key | LSB |
| 921 | KEY_ALM | Stores the status of ALM key | LSB |
| 922 | KEY_UP | Stores the status of ▲ key | LSB |
| 923 | KEY_DOWN | Stores the status of ▼ key | LSB |
| 924 | KEY_LEFT | Stores the status of ◀ key | LSB |
| 925 | KEY_RIGHT | Stores the status of ▶ key | LSB |

| Address | Name | Description | Remark |
| --- | --- | --- | --- |
| 926 | KEY_SET | Stores the status of SET key | LSB |
| 927 | KEY_ENT | Stores the status of ENT key | LSB |
| 928 | KEY_F1 | Stores the status of F1 key | LSB |
| 929 | KEY_F2 | Stores the status of F2 key | LSB |
| 930 | KEY_F3 | Stores the status of F3 key | LSB |
| 931 | KEY_F4 | Stores the status of F4 key | LSB |
| 932 | W_ALWAYS_ON | Always On( 0x0001 ) |  |
| 933 | W_ALWAYS_OFF | Always Off( 0x0000 ) |  |
| 934 | W_SCAN_TOGGLE | Scan toggle( 0x0001 <-> 0xFFFE ) |  |
| 935 | W_FATAL_ERROR | Turns on when a fatal error has occurred | LSB |
| 936 | W_LIGHT_ERROR | Turns on when an ordinary error has occurred. | LSB |
| 937 | W_ERROR_KIND | Stores the error code |  |
| 938 | W_RTC_DATA_ERROR | Stores the error code of RTC |  |
| 939 | W_WDT_COUNT | Not used |  |
| 940 | W_20MS | 20-ms cycle clock | LSB |
| 941 | W_100MS | 100-ms cycle clock | LSB |
| 942 | W_200MS | 200-ms cycle clock | LSB |
| 943 | W_1S | 1-sec cycle clock | LSB |
| 944 | W_2S | 2-sec cycle clock | LSB |
| 945 | W_10S | 10-sec cycle clock | LSB |
| 946 | W_20S | 20-sec cycle clock | LSB |
| 947 | W_60S | 60-sec cycle clock | LSB |
| 948 | WDT_PTR_H | The high address of WDT error |  |
| 949 | WDT_PTR_L | The low address of WDT error |  |
| 950 | CALC_L_ERR | The error flag of calculation(divided by zero) | Calculation 0 ~ 15 |
| 951 | CALC_H_ERR | The error flag of calculation(divided by zero) | Calculation 16~19 |
| 952~999 | RESERVED | - |  |