#### Chapter 1 Overview

1.1 How to use the User’s Manual

This User’s Manual provides the information such as product specification, performance and operation method needed to use PLC System composed of Smart I/O module.

The User’s Manual is composed of as follows.

CHAP.1 Overview

Here describes the configuration of the user’s manual, product characteristics and terminology.

CHAP.2 Product Specification

Here describes common specification of each product used for Smart I/O series.

CHAP.3 System Configuration

Here describes the kinds of product available for Smart I/O series and system configuration method.

CHAP.4 Communication Programming

Here describes common communication program operation method to act Smart I/O module.

CHAP.5 Profibus-DP Communication

Here describes basic communication method of Profibus-DP(Pnet) communication module.

CHAP.6 DeviceNet Communication

Here describes basic communication method of DeviceNet(Dnet) communication module.

CHAP.7 Rnet Communication

Here describes basic communication method of FIELDBUS(RNET) communication module.

CHAP.8 Modbus Communication

Here describes basic communication method of Modbus(Snet) communication module.

CHAP.9 Installation and Wiring

Here describes installation and wiring method, and notices to make sure of the reliability of PLC system.

CHAP.10 Maintenance and Repair

Here describes check list and method to run PLC system normally for a long term.

CHAP.11 Trouble Shooting

Here describes various errors to be occurred while using the system and the action to solve the problem.

Appendix

Here describes the product terminology and external dimension for system installation.

1.2 Characteristics of Smart I/O

1) The characteristics of Smart I/O series is as follows.

- Product design based on International Electrotechnical Commission (IEC 61131) (GLOFA

series in common)

 Easy support to programming device

 Standard language (IEC 61131-3) provided (IL / LD / SFC)

(2) Open network by selecting international standard communication protocol.

(3) Available to communicate with remote master module independently without power module/CPU module.

(4) Available to set maximum 32 ~ 64 stations.

- Maximum 64 stations (Rnet,Dnet)

- Maximum 32 stations (Pnet,Snet)

(5) Enables to save the cost for installation and maintenance.

(6) Various system configuration and simple maintenance and repair.

(7) Easy to change the system.

(8) Compatible with other maker’s product.

- Available to connect Smart I/O to other maker’s master.

(9) Easy to set the system as the station address setting by hardware is available.

(10) Simple communication programming.

- Using the GMWIN/KGLWIN  high speed link parameter

- But for Modbus, using function block

(11) Supports various I/O.

- DC input 16/32 points, TR output 16/32 points, Relay output 16 points

(12) Supports various OPEN type communication method.

- Profibus-DP, DeviceNet, Rnet, Modbus

(13) Easy to configure and use as integrating PLC.

- Packaged by one unit including CPU, I/O and communication function.

(14) Provides the online network status detection function.

- Available to know the remote module status through high speed link monitor.

(15) Supports high speed communication.

(16) Flexible communication relation is available as the speed shall be set automatically

according to the speed of master.

- Profibus-DP, DeviceNet etc.

1.3 Product Configuration of Smart I/O

1.3.1 Type Name Indication

| Classification |  | Model name | Type name indication |
| --- | --- | --- | --- |
| DC Input | 16 point | GPL-D22A | G L A Communication type P : Profibus-DP D : DeviceNet R : Rnet S : Modbus I/O Type D2 : DC24V input RY : Relay output TR : TR output DT : DC input/ TR output I/O points 2 : 16 points 4 : 32 points G L : Communi cation Char. class |
|  |  | GDL-D22A |  |
|  |  | GRL-D22A |  |
|  |  | GSL-D22A |  |
|  | 32 point | GPL-D24A |  |
|  |  | GDL-D24A |  |
|  |  | GRL-D24A |  |
|  |  | GSL-D24A |  |
| TR output | 16 point | GPL-TR2A |  |
|  |  | GDL-TR2A |  |
|  |  | GRL-TR2A |  |
|  |  | GSL-TR2A |  |
|  | 32 point | GPL-TR4A |  |
|  |  | GDL-TR4A |  |
|  |  | GRL-TR4A |  |
|  |  | GSL-TR4A |  |
| RY Output | 16 point | GPL-RY2A |  |
|  |  | GDL-RY2A |  |
|  |  | GRL-RY2A |  |
|  |  | GSL-RY2A |  |
| DC/TR combined | 32 point | GPL-DT4A |  |
|  |  | GDL-DT4A |  |
|  |  | GRL-DT4A |  |
|  |  | GSL-DT4A |  |

1.3.2 Characteristics per Module

| Classification | Product Name | Type name | Specification |
| --- | --- | --- | --- |
| Smart I/O | Profibus-DP | GPL-TR2A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 16 points |
|  |  | GPL-TR4A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 32 points |
|  |  | GPL-DT4A |  Input voltage(power section) : DC24V(rating)  I/O point(combined) : DC input 16 points/ TR output 16 points |
|  |  | GPL-RY2A |  Input voltage(power section) : DC24V(rating)  I/O point : Relay output 16 points |
|  |  | GPL-D22A |  Input voltage(power section) : DC24V(rating)  I/O point : DC input 16 points |
|  |  | GPL-D24A |  Input voltage(power section) : DC24V(rating)  I/O point : DC input 32 points |
|  | DeviceNet | GDL-TR2A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 16 points |
|  |  | GDL-TR4A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 32 points |
|  |  | GDL-DT4A |  Input voltage(power section) : DC24V(rating)  I/O point(combined) : DC input 16 points/ TR output 16 points |
|  |  | GDL-RY2A |  Input voltage(power section) : DC24V(rating)  I/O point : Relay output 16 points |
|  |  | GDL-D22A |  Input voltage(power section) : DC24V(rating)  I/O point: DC input 16 points |
|  |  | GDL-D24A |  Input voltage(power section) : DC24V(rating)  I/O point: DC input 32 points |
|  | Modbus | GSL-TR2A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 16 points |
|  |  | GSL-TR4A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 32 points |
|  |  | GSL-DT4A |  Input voltage(power section) : DC24V(rating)  I/O point(combined) : DC input 16 points/ TR output 16 points |
|  |  | GSL-RY2A |  Input voltage(power section) : DC24V(rating)  I/O point : Relay output 16 points |
|  |  | GSL-D22A |  Input voltage(power section) : DC24V(rating)  I/O point : DC input 16 points |
|  |  | GSL-D24A |  Input voltage(power section) : DC24V(rating)  I/O point : DC input 32 points |

| Classification | Product name | Type name | Specification |
| --- | --- | --- | --- |
| Smart I/O | Rnet | GRL-TR2A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 16 points |
|  |  | GRL-TR4A |  Input voltage(power section) : DC24V(rating)  I/O point : Transistor output 32 points |
|  |  | GRL-DT4A |  Input voltage(power section) : DC24V(rating)  I/O point(combined) : DC input 16 points / TR output 16 points |
|  |  | GRL-RY2A |  Input voltage(power section) : DC24V(rating)  I/O point : Relay output 16 points |
|  |  | GRL-D22A |  Input voltage(power section) : DC24V(rating)  I/O point : DC input 16 points |
|  |  | GRL-D24A |  Input voltage(power section) : DC24V(rating)  I/O point: DC input 32 points |

1.3.3 Version Compatibility Table per Smart I/O Module

The followings describe the list that enables the compatibility of O/S version between various

CPU and master module to use Smart I/O series.

Before using the product, please refer to the list below and apply it to system configuration.

| Classification | Model type | Available O/S |
| --- | --- | --- |
| CPU | GM3 | More than Version 2.5 |
|  | GM4 | More than Version 2.6 |
|  | GM6 | More than Version 1.9 |
|  | GM7 | More than Version 1.6 |
|  | K1000S | More than Version 3.1 |
|  | K300S | More than Version 3.1 |
|  | K200S | More than Version 2.2 |
|  | K80S | More than Version 1.6 |
| PADT | GMWIN | More than Version 3.6 |
|  | KGLWIN | More than Version 3.3 |
| Smart I/O Master | Profibus-DP | More than Version 1.0 |
|  | DeviceNet | More than Version 1.2 |
|  | Rnet | More than Version 1.0 |
|  | Modbus | More than Version 2.0 |