# Chapter 1. Introduction

1.1 Features

1) GLOFA-GM series features:

(1) Design on the basis of international standard specifications(IEC 1131-3)

 Easy programming device support.

 Languages in compliance with IEC1131-3 are given. (IL/ LD / SFC)

(2) Open network by use of communications protocol in compliance with international

standard specifications.

(3) High speed processing with an operation-dedicated processor included.

(4) Various special modules that enlarge the range of application of the PLC.

2) GM6-CPUA/B/C features :

(1) High speed operation processing

High speed processing of 0.5㎲/step with an operation-dedicated processor included.

(2) Heightened Self-diagnosis

Cause of errors is easily found as error codes has been more divided in accordance with

their contents.

(3) Restart mode setting

The User can set Cold / Warm restart mode in accordance with the environment.

(4) Debug operation

On-line debugging is available if the PLC operation mode is set to debug operation mode.

Debugging functions :

 Executed by one instruction.

 Executed by the break-point settings

 Executed by the device status

 Executed by the specified scan times

(5) Various Program Executions

Time driven task, external and internal contact task programs as well as scan program can

be executed by setting the execution condition. The user can set variously the program

execution mode.

(6) Various data type

1.2 Specifications

The following shows the general specifications of the GLOFA-GM series.

| Items |  |  |  | Specifications |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | GM6-CPUA | GM6-CPUB | GM6-CPUC |  |
| Operation method |  |  |  | Cyclic operation of stored program, Interrupt task operation |  |  |  |
| I/O control method |  |  |  | Scan synchronized batch processing method(Refresh method) |  |  |  |
| Programming language |  |  |  | Ladder Diagram(LD) Instruction List(IL) Sequential Function Chart(SFC) |  |  |  |
| Number of instructions |  | Operator |  | LD : 13, IL : 21 |  |  |  |
|  |  | Basic function |  | 194 |  |  |  |
|  |  | Basic function block |  | 11 |  |  |  |
|  |  | Special function block |  | Each special module have their own special function blocks |  |  |  |
| Processing speed |  | Operator |  | 0.5㎲/step |  |  |  |
|  |  | Basic function |  |  |  |  |  |
|  |  | Basic function block |  |  |  |  |  |
| Programming memory capacity |  |  |  | 68 k bytes(17 k steps) |  |  |  |
| I/O points |  |  |  | 256 points |  |  |  |
| Data memory |  | Direct variable area |  | 2 to 8 k bytes |  |  |  |
|  |  | Symbolic variable area |  | 30 k bytes – Direct variable area |  |  |  |
| Timer |  |  |  | No limitations in points. Time range : 0.01 to 4294967.29 sec(1193 hours) |  |  | 1 point occupies 20 bytes of symbolic variable area. |
| Counter |  |  |  | No limitations in points Counting range: -32768 to +32767 |  |  | 1 point occupies 8 bytes of symbolic variable area. |
| Program types | Numbers of program blocks |  |  | 100 |  |  |  |
|  | Initialization programs |  |  | 1 (_INIT) |  |  |  |
|  | Task Programs |  | Time driven tasks | 0 ~ 8 |  |  | Total : 8 (The type of task is variable, however, total numbers of tasks is 8.) |
|  |  |  | External interrupt tasks | 0 ~ 8 |  |  |  |
|  |  |  | Internal task | 0 ~ 8 |  |  |  |
| Operation modes |  |  |  | RUN, STOP, PAUSE and DEBUG |  |  |  |
| Restart modes |  |  |  | Cold, Warm |  |  |  |
| Self-diagnostic functions |  |  |  | Watch dog timer, Memory error detection, I/O error detection, Battery error detection, Power supply error detection, etc. |  |  |  |
| Data protection method at power failure |  |  |  | Set to 'Retain' variables at data declaration. |  |  |  |
| Built-in special functions |  |  |  | RS-232C | RS-422/485 RTC PID control | RS-232C RTC PID control High Speed Counter |  |
| Internal current consumption |  |  |  | 170mA | 210mA | 170mA |  |
| Weight |  |  |  | 0.11Kg | 0.11 Kg | 0.12Kg |  |