Chapter 3. Assign I/O address

To read data I/O ,the CPU assigns I/O address (%I area) to each input modules

To write data I/O, the CPU assigns I/O address (%Q area) to each output modules

Power module

CPU module

Input (16 points)

Input (32 points)

Input (64 points)

Output (16 points)

Output (32 points)

A/D module(4ch)

D/A module (8ch)

Output (32 points)

Main base (8 slots)

%Q0.7.0~%Q0.7.31

%Q0.4.0~%Q0.4.31

%Q0.3.0~%Q0.3.15

%I0.2.0~%I0.2.63

%I0.1.0~%I0.1.31

%I0.0.0~%I0.0.15

- The following shows I/O No. allocation method.

Input  : %  I   0.  0.  0

Output :%  Q  0.  1.  15

Contact number on I/O module

0 ~ 63

Slot number of Base

0 ~ 7

Base number.  **1

0 ~1(3)

**1 : Base number is upto 1 for GLOFA-GM6, 3 for GLOFA-GM4

Remark

- I/O No. allocation means to give an address to each module in order to read data from input modules and output data to output modules.

- Fixed 64 points are allocated to each module for I/O points.

- Fixed 64 points are allocated regardless of mounting/dismounting